INSTRUCTION MANUAL

ELECTROSTATIC DISCHARGE SIMULATOR

ESS-S3011

Remote Control Software

NOISE LABORATORY CO., LTD.

Edition 1.01

AEC00265-00E-0B

NOTICE

This product is copyrighted by Noise Laboratory Co., Ltd. It is strictly prohibited to reproduce, transfer or alter the product without the permission of its copyright holder, or use it for any purpose other than testing with the Electrostatic Discharge Simulator ESS-S3011.

Copyright(C) 2012 Noise Laboratory Co., Ltd. All Rights Reserved.

•	The contents of this booklet are subject to change without prior notice.

•	No part of this booklet may be reproduced or transferred, in any form, for any purpose, without the permission of Noise Laboratory Co., Ltd.

•	The contents of this booklet have been thoroughly checked. However, if a doubtful point, an error in writing or a missing is found, please contact us.

•	In principle, one set of the product shall be purchased for each computer and installed to that computer only.

•	Noise Laboratory Co., Ltd. shall have no liability for any trouble resulting from the misuse or improper handling of this product regardless of the contents of this booklet or arising from the repair or remodeling of this product by a third party other than Noise Laboratory Co., Ltd. or its authorized person.

•	The product is subject to program improvement and modification without prior notice.

•	It is prohibited to reverse-engineer, decompile or disassemble the product.

•	Exit all other running applications before starting this application.

•	In no event shall Noise Laboratory Co., Ltd. be liable for any results arising from the use of this product.

Company names and product names that appear in this document are generally trademarks or registered trademarks of their respective companies. This document may not use the ® and TM symbols.

# IMPORTANT SAFETY PRECAUTIONS

This software is used to control the Electrostatic Discharge Simulator ESS-S3011 remotely. The following are important precautions for safe handling of ESS-S3011, when controlling it using this product. Read and understand them before use.

Also read the contents of this document and the instruction manual for the ESS-S3011 main unit before using the product.

- The instrument may not be used in a location where fire is prohibited or there is a risk of explosion. Failure to follow this rule risks igniting a fire due to an electrical discharge.

- The instrument may not be used by people fitted with electronic medical devices such as pacemakers and such people may not enter the testing site while the instrument is operating.

- Before connecting the instrument, turn off the power to the main unit, supply voltage, the simulator to be connected and equipment under test (hereinafter “EUT”), and make sure that no electricity is being supplied.

- Before setting up the test site, connecting the equipment, or starting testing, please read the Chapter entitled "Basic Precautions for Safe Use" which contains additional safety advice.

- Do not turn off the power to the main unit or unplug the connection cable, while this software is still running. Failure to follow this rule may make the operation of the PC unstable or cause the OS to operate improperly. Always exit this software before turning off the power to the main unit.

# APPLICATION FORM FOR INSTRUCTION MANUAL

To: Noise Laboratory Co., Ltd. via sales agent

|  | We place an order for an instruction manual. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Model Name | Remote Software for ESS-S3011 |  |  |  |  |  |  |  |  |  |  |  |
|  | Serial No. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Applicant Address |  |  |  |  |  |  |  |  |  |  |  |  |
| Cut Line |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Company Name |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Department |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Contact Person |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Phone No. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | FAX No. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cut off this page "PURCHASE ORDER FOR INSTRUCTION MANUAL" from this volume and keep it for future use with care. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | When an INSTRUCTION MANUAL is required, fill in the above Application Form and mail or fax it to your nearest sales agent of Noise Laboratory or Noise Laboratory. |  |  |  |  |  |  |  |  |  |  |  |  |
| Cut Line |  |  |  |  |  |  |  |  |  |  |  |  |  |

# TABLE OF CONTENTS

1.	IMPORTANT SAFETY PRECAUTIONS	1

2.	APPLICATION FORM FOR INSTRUCTION MANUAL	3

3.	TABLE OF CONTENTS	5

4.	PREFACE	6

4.1.	Preface	6

4.2.	About The Product	6

4.3.	Product Features	6

5.	BASIC PRECAUTIONS FOR SAFE USE	7

5.1.	Danger Notice Signs and Meanings	7

5.2.	Basic Safety Precautions	7

6.	SETUP AND STARTUP	8

6.1.	Setup	8

6.2.	Startup	20

7.	OPERATION	21

7.1.	Main Screen	21

7.1.1.	Test Condition Settings	22

7.1.2.	Test Environment Settings	24

7.1.3.	Execute	25

7.2.	Others	28

7.2.1.	File Menu	28

7.2.2.	Option Menu	29

7.2.3.	Version Information Menu	29

8.	OPERATING ENVIRONMENT	30

9.	SUPPORT	31

10.	CONTACTING TECHNICAL SUPPORT	32

# PREFACE

## Preface

We thank you very much for purchasing ESS-S3011. It is recommended that the contents of this manual be thoroughly understood and used as a ready reference for operation.

This manual uses two different types of parentheses to distinguish button names from other identifiers and messages displayed on the screen: <button name> and [other identifiers and messages displayed on the screen].

Also read the instruction manual for the ESS-S3011 main unit.

- This Instruction Manual was prepared so that any person who can observe the prescribed instruction method and operating precautions may safely handle and fully utilize the electrostatic simulator ESS-S3011 and the software.

- Keep this Instruction Manual and the instruction manual for the ESS-S3011 main unit by your side or other proper location so that they may be readily available when using the ESS-S3011.

## About The Product

This software is used to run, control and assist ESS-S3011, which is a simulator manufactured by Noise Laboratory Co., Ltd.

The simulator is connected to the PC with an optical cable using the optional USB optical module kit (MODEL: 07-00022A).

## Product Features

- A personal computer can be used to control ESS-S3011.

- It makes it easier to set test conditions based on the IEC61000-4-2 standard.

- It enables electrically-insulated remote control, as the simulator is connected to the PC with an optical cable by connecting the optical interface unit to a USB port of the PC.

- The function to save test conditions allows you to perform tests under the same conditions, whenever required.

- The function to support report creation allows you to output test conditions on MS-Excel.
(MS-Excel2000 or later must be installed before using the report creation support function.)

# BASIC PRECAUTIONS FOR SAFE USE

## Danger Notice Signs and Meanings

Indicates a location where a high voltage or large current is present and may be dangerous to human bodies.

Indicates that careful handling is required.

Refer to this instruction manual to protect human bodies and devices.

## Basic Safety Precautions

The following precautions apply to the control of ESS-S3011 using this product. In addition, the instruction manual for the main unit also provides safety precautions for each device.
Comply with the precautions when using the product.

- Noise Laboratory Co., Ltd. and affiliated sales agencies have no liability for any human-related accidents or property damage due to the operation of the software and any resultant further damage. [Precaution related to human bodies, operations, the environment and connections]

- Incorrect or careless operation may result in fatal wounds. [Precaution related to human bodies, operations, the environment and connections]

- The simulator may not be operated by people fitted with electronic medical devices such as pacemakers and such people may not enter the testing site while the simulator is operating. [Precaution related to human bodies and operations]

- Avoid using the simulator in humid or dusty locations. [Precaution related to the environment]

- If any repair, maintenance or internal adjustment is required, it must be performed only by a service engineer who has relevant qualification. [Precaution related to handling and safety]

# SETUP AND STARTUP

## Setup

(1) Before setup

ESS-S3011 manages the internal supply power including the communication function with the power switch at the back.

Note:	The operation of PC may become unstable, if the power to the device is switched off when the software is still running. Always exit the software before turning off the device.

(2) Setting up the software

	Log on with the ID that has Administrator privileges for Windows.

	Insert the setup CD-ROM into the CD-ROM drive and select the [English] folder from the [ESS-S3011] folder.

	Run the (Setup.exe) in the folder or (Setup_EssS3011_English.msi).

	The installation program will start. Follow the instructions on the screen to perform the installation.

	Once the installation is completed, select [start] - [All Programs(P)] to check that [ESS-S3011Eng] is registered to [NoiseKen] - [ESS-S3011Eng].

This completes the setup of the software.

(3) Installing the driver (WindowsXP)

The driver must be installed to run this software.

	Log on with the ID that has Administrator privileges for Windows, and insert the setup CD-ROM into the CD-ROM drive.

	Connect the optical interface unit to a USB port of the PC.

Click



Click

When Windows recognizes that there is new hardware, the following screen will appear.
Select [Install from a list or specific location] and click the <Next> button.












If the above screen does not appear, check the connection between the PC and the optical interface unit once again.
If a drive with a large capacity is connected to the network drive, it may take several minutes for the above screen to appear.

	Select the checkbox for [Search for the best driver in these locations.] and then [Search removable media]. Click the <Next> button.

Click

Click

Click

Click

	Although the following screen appears, click the <Continue Anyway> button.

	When [Completing the Found New Hardware Wizard] appears, click the <Finish> button.

	Then, [Found New Hardware Wizard] appears. Select [Install from a list or specific location] and click the <Next> button.

Click

Click

Click

Click

Click

	Select the checkbox for [Search for the best driver in these locations.] and then [Search removable media]. Click the <Next> button.

Click

	Although the following screen appears, click the <Continue Anyway> button.

	When [Completing the Found New Hardware Wizard] appears, click the <Finish> button.

If the optical interface unit is reconnected to a different USB port from the current port, the driver may need to be reinstalled.

(4) Checking the installation (WindowsXP)

Click

	Select [start]-[My Computer], and click [View system information].

Click

Click

	When [System properties] appears, select the [Hardware] tab and click the <DeviceManager> button.

Check

	Check to make sure that [Universal Serial Bus controllers] shows [USB Serial Converter] and it is not marked with “!” or “”.

Check

	Check to make sure that [Ports] shows [USB Serial Port] and it is not marked with “!” or “”. (The number following COM varies depending on the PC environment.)

(5) Installing the driver (Windows7)

The driver must be installed to run this software.

As Windows7 does not display the new hardware search wizard, it must be installed manually from the control panel.

	Log on with the ID that has Administrator privileges for Windows, and insert the setup CD-ROM into the CD-ROM drive.

	Connect the optical interface unit to a USB port of the PC.

Click

Click

	Click the <Start> button, and click [Control Panel].

Click

	In [Control Panel], click [System and Security].

	In [System], click [Device Manager].

Right-click

Click

Click

	If [Other devices] shows [USB <-> Optical Serial Cable] marked with “!”, right-click [Update Driver Software...] with the mouse.

Click

	When the [Update Driver Software - USB <-> Optical Serial Cable] screen appears, click [Browse my computer for device driver software].

	Specify the name of the drive into which you inserted the driver CD, and select the checkbox for [Include subfolders].

Enter CD drive name or click <Browse...> to select the drive

Select the box

Click

Click

	When the [Windows Security] screen appears, click [Install this driver software anyway].

	Once the USB Serial Converter is installed successfully, the completion screen appears.

Click

Install the driver software for the USB Serial Port.
If [Other devices] shows [USB Serial Port] marked with “!”, right-click [Update Driver Software...] with the mouse.

Right-click

Click

Click

When the [Update Driver Software - USB Serial Port] screen appears, click [Browse my computer for driver software].

Enter CD drive name or click <Browse...> to select the drive

Select the box

Click

Specify the name of the drive into which you inserted the driver CD, and select the checkbox for [Include subfolders].

When the [Windows Security] screen appears, click [Install this driver software anyway].

Click

Click

Once the USB Serial Port is installed successfully, the completion screen appears.

(6) Checking the installation (Windows7)

Click

Click

	Click the <start> button, and click [Control Panel] in [start menu].

Click

	In [Control panel], click [System and Security].

	In [System and Security], click [Device Manager].

Make sure it's not marked with "!"

Expand

Expand

Click



Make sure it's not marked with "!"

In [Device Manager], expand [Ports] and check to make sure that [USB Serial Port(COMx)] is not marked with “!” or “”.
(The number following COM varies depending on the PC environment.)
Also expand [Universal Serial Bus controllers] and check to make sure that [USB Serial Converter] is not marked with “!” or “”.

(7) Uninstallation (WindowsXP)

To uninstall the software, select [start] - [My Computer] on the taskbar, and click [Add or remove programs]. Select [ESS-S3011Eng] from the displayed list, and click the <Remove> button. Follow the instructions on the screen.

To uninstall the driver, select [start] - [My Computer], and click [Add or remove Programs]. Select [FTDI USB Serial Converter Driver] from the displayed list, and click the <Remove> button. Follow the instructions on the screen to continue the uninstallation.

(8) Uninstallation (Windows7)

To uninstall the software, select [start]-[Control Panel] on the taskbar, and click [Add or remove Programs]. Click [ESS-S3011Eng] from the displayed list. When the user account control appears, click <YES>. Follow the instructions on the screen to continue the uninstallation.

(9) Connecting to ESS-S3011

	Connect the USB cable of the optical interface unit to the PC.

	Connect the optical cable that comes with the optical interface unit to the REMOTE control port at the front of the device and the optical interface unit.

	Turn on the power to the device.

	Start the software.

This completes the connection procedure.

When starting it, always turn on the power to the simulator before starting the software.

(10) Exiting the software and turning off the power

	Exit the software.

	Turn off the power to the device.

When exiting it, always exit the software before turning off the power to the device.

When disconnecting the connection cable, also exit the control software before disconnecting it.

If the device is turned off or the connection cable is disconnected without exiting the software, the operation of the PC may become unstable or the OS may stop running.

This section uses the category display of Windows XP for screen examples and operating methods. Depending on your OS version and settings, the screens and operating methods may differ.

## Startup

Click [start] - [All Programs] - [NoiseKen] - [ESS-S3011], and select [ESS-S3011] to start the software.

After it is started, check the connection with ESS-S3011. The main screen (explained in the next chapter) will appear.

If there is a communication error, the following model name selection screen appears.

		

[Connection Error] Screen

 <Restart> button

Starts the software without communicating with ESS-S3011.

 <Retry> button

Checks the communication with ESS-S3011 again.

 <Exit> button

Stops starting up the software.

*	When <Restart> is selected, the software is started without allowing any test to be performed. Use this option to check test conditions or settings only.

When connecting to ESS-S3011 after this application is started, check to make sure that ESS-S3011 is turned on. Then select [Option] - [Connect...] to open the simulator connection status dialog box.

The box shows <Connect> when it is successfully connected to ESS-S3011, and shows <None> when it is not connected. The connection takes several seconds to be established, as it attempts to communicate with the simulator.

Simulator Connection Status Dialog Box

# OPERATION

## Main Screen

In this software, the smallest testing unit is called “Step”.

The following screen is the main screen that shows test conditions.



		



Main Screen

 <Test...> button

Opens the test environment dialog box and the test execution dialog box.

 <Settings...> button

Opens the test condition settings dialog box.

 <Add...> button

Adds tests to the [Test No.] list.

 <Delete...> button

Deletes tests registered on the [Test No.] list.

 <Test No.> list

Displays test details.

### Test Condition Settings

Test conditions are set.

		

	







[Test condition settings] Screen

 Disc. Method

Selecting <CONTACT> performs a contact discharge test.

Selecting <AIR> performs an air discharge test.

 IEC LEVEL

In [IEC LEVEL], different voltages are set between contact and air discharge tests.

| IEC LEVEL | Contact discharge (CONTACT) | Air discharge (AIR) |
| --- | --- | --- |
| IEC LEVEL 1 | 2kV | 2kV |
| IEC LEVEL 2 | 4kV | 4kV |
| IEC LEVEL 3 | 6kV | 8kV |
| IEC LEVEL 4 | 8kV | 15kV |
| IEC LEVEL X | Any voltage | Any voltage |

 Trig.

Discharge triggers are selected.

Selecting <ESS> enables the trigger button on the ESS-S3011 main unit.

Selecting <GUN> enables the trigger button on the discharge gun.

Selecting <EXT.> enables an external trigger.

Whichever trigger is selected, the trigger button on the remote software screen is enabled to discharge.

 Polarity

Selecting <+> outputs a + waveform.

Selecting <-> outputs a - waveform.

Selecting <+/-> discharges a + waveform for the specified number of times, and then outputs a – waveform.

(<+/-> appears, only when the checkbox for [SWEEP] is selected.)

 SWEEP

When the checkbox is selected, <+/-> appears in the [Polarity] dropdown list and [STEP] and [END] appear in [Voltage].

 Voltage

Test voltage is set within the range from 0.20kV to 30.5kV, and in steps of 0.01kV.

([STEP] and [END] appear, only when the checkbox for [SWEEP] is selected.)

 Interval (it can be set only for contact discharge)

The discharge interval is set within the range from 0.05 sec to 600 sec.

It is set in steps of 0.01 sec from 0.05 sec to 1 sec. It is set in steps of 0.1 sec from 1 sec to 600 sec.

The discharge interval cannot be set for air discharge.

 Count

The discharge count is set within the range from 1 to 60000 times, and in steps of 1 time.

### Test Environment Settings

The test environment is set.

The settings are reflected on the contents of the report.

If the settings are not saved by pressing the <Save> button, the described contents are cleared when the software is exited.

[Test environment] Screen

| Date | Press the <Test...> button on the main screen and display the date on which the preparation for starting the test was completed. The test date cannot be changed. |
| --- | --- |
| Time | Press the <Test...> button on the main screen and display the time at which the preparation for starting the test was completed. The test time cannot be changed. |
| Temperature | Enter the test environment temperature. |
| Humidity | Enter the test environment humidity. |
| Motel/Type | Enter the model name/type of the EUT. |
| Serial No. | Enter the serial number of the EUT. |
| Operator | Enter the name of the operator. |
| Standard | Enter the name of the standard. |
| <Save> button | The entered details can be saved. The entered details are used as future default values. However, the test date and test time are not saved. |

### Execute

The simulator is controlled.

It performs the test that is set on the test setting screen.









	







[Execute] Screen

 Step No.

Displays the test step number.

 Interval

It functions, only when the contact discharge is selected.

Discharge occurs, when it counts down to 0.

It outputs only the first discharge in the minimum time, regardless of the discharge interval setting.

 Count

It counts up from 0. The test is finished, when the specified count is reached.

 <START> button

It turns on the high-voltage power to enter the discharge trigger wait state.

When the high-voltage power is turned on, the <TRIG> button is enabled.

 <STOP> button

It turns off the high-voltage power to enter the discharge-disabled state.

 <TRIG> button

It turns on or off the discharge trigger.

| Contact discharge (CONTACT) | Continues to discharge for the specified number of times, based on the set discharge interval. Pressing the <TRIG> button again suspends the discharge. |
| --- | --- |
| Air discharge (AIR) | While <TRIG> is on, the discharge tip at the end of the discharge gun remains charged. Pressing the <TRIG> button again terminates the discharge and counts up the discharge count. |

 Test execution list

It displays the contents of the test settings.

When any [Step No.] is clicked while the test is stopped, it can be executed starting from that test.

Double-clicking any [Step No.] allows you to edit its result and comment.

 Test result entry field

It allows you to change the result of the test displayed in [Step No.].

It can also be used for writing a memo during the execution of the test.

When the checkbox for <Test result A is automatically entered at the completion of step.> is selected, the result A is automatically entered, every time the Step is completed during the execution of the test.

 Batch result setting and report creation

Clicking the <Set All Results> button allows all test results to be set at once.

Clicking the <Del. All Comments> button allows all comments to be deleted at once.

Clicking the <Create Reports...> button starts MS-Excel and displays test details.

(MS-Excel must be installed to the PC beforehand.)

Tables are created in a plain form without any change to the CELL format for easier processing. Therefore, process them as required before use.

* Use the functions of MS-Excel to save and print test results, if needed.

## Others

This section explains other functions.

### File Menu

| New | Creates a new test window. |
| --- | --- |
| Open... | Opens saved test settings. |
| Close | Closes the test setting window that is currently displayed. |
| Save | Overwrites and saves the test setting window that is currently being edited. |
| Save As... | Saves the test setting window that is currently being edited by adding a new name. |
| Exit | Exits the application. |

### Option Menu

| Option Settings... | Opens the option settings screen. The options to be entered include a company name, test location, simulator model name, simulator serial No., discharge gun model name, discharge gun serial No., discharge cup model name, discharge cup serial No., CR unit model name, and CR unit serial No. The entered details are reflected on the report. The contents of the option settings are saved to the system drive. |
| --- | --- |
| Information... | Opens the test environment settings screen. The information to be entered for the test environment includes temperature, humidity and test target. For more details, refer to the [Test environment] Screen. The contents of test environment settings are saved to the system drive, only when the <Save> button is pressed. If the settings are not saved by pressing the <Save> button, they are cleared when the software is exited. |
| Connect... | Checks the status of the connection with ESS-S3011. |

### Version Information Menu

It displays the version confirmation screen of the application.

# OPERATING ENVIRONMENT

| Computer | : | PC/AT-compatible machine |
| --- | --- | --- |
| CPU | : | Clock frequency: 1GHz or more |
| Main memory | : | 2GB or more |
| HDD | : | Free space: 32MB or more (for the installation and startup of the application) |
| OS | : : | Microsoft® Windows®XP Operating System (32-bit) (English version) Microsoft® Windows®7 Operating System (32-bit) (English version) |
| Graphics | : | SXGA (1280*1024 : 32767 colors) or higher supported |

In addition to the above, the following conditions also apply:

- USB optical module kit (Model: 07-00022A) is available (optional product).

- An unused USB port is available.
(One port is required.)

- A pointing device such as a mouse is available.

- A CD-ROM or DVD-ROM drive is available.

- The PC is connected to the simulator before the remote control program is started.
(If not connected, no communication can be established.)

- In case of using the report creation support function, MS-Excel2000 or later must be installed beforehand.

# SUPPORT

The free-of-charge support period of this product is one year after the delivery.

Either of the following cases, however, may incur the actual cost:

1:	fault or damage due to careless handling (including the software itself and media)

2:	Fault or damage due to an earthquake, fire or other natural disasters

In any of the following cases, support requests may be rejected:

- Product to which software has been changed or altered without the permission of Noise Laboratory

- Product whose sale was discontinued 5 years or more ago

- Product that was delivered 8 or more years ago

# CONTACTING TECHNICAL SUPPORT

- If you experience a malfunction, please have available both the model and serial number of your unit and contact the nearest distributor/agent or Noise Laboratory Technical Support.

- When it is necessary to send your unit back to Noise Laboratory, find its model name and serial number and complete the repair order form, including the status and symptoms of the problem and details of your request.

	Repair and Calibration Center

TEL (0088)25-3939 (toll free) / (042)712-2021

FAX (042)712-2020

NOISE LABORATORY CO., LTD.

Produced by: Noise Laboratory Co., Ltd.

1-4-4, Chiyoda, Chuo-ku, Sagamihara City, Kanagawa Pref., 252-0237, Japan

TEL: +81-(0)42-712-2051 FAX: +81-(0)42-712-2050

URL: http://www.noiseken.co.jp

Printed in Japan